OAS Agricultural Data Logger Hardware Datasheet — Rev 2 ESP32-S3 Asset Tracker · LoRa + GPS · Vehicular Power

California Strawberry Commission

Operator Aid System (OAS) — Field-Ready Prototype

Hardware Engineer: Alex Herman

Document Date: April 20, 2026

Revision: DRAFT (reverse-engineered from PCB Rev2 source)

# Table of Contents

A generated TOC follows on the next page. If it does not populate automatically, right-click the entry below in Word and select "Update Field".

# 1. Overview

The OAS Agricultural Data Logger Rev 2 is a ruggedized, low-power asset-tracker PCB designed for deployment on agricultural vehicles (tractors, spray rigs, ATVs). It continuously logs high-precision GNSS position and auxiliary sensor data to onboard microSD storage and reports uplink via LoRa long-range radio and — when in Wi-Fi/Bluetooth range — via the ESP32-S3's native wireless stack. The board accepts either 12–14 V tractor power or USB 5 V, and supports a rechargeable single-cell Li-ion pack for graceful-shutdown power backup.

### Key Features

- MCU: Espressif ESP32-S3-MINI-1U-N8 (dual-core Xtensa LX7 @ 240 MHz, 8 MB flash, Wi-Fi 2.4 GHz, BLE 5, U.FL antenna)

- Radio: Seeed Wio-SX1262 LoRa module (Semtech SX1262, sub-GHz long-range transceiver), on SPI with full BUSY/DIO1/RST/RF_SW1 control

- GNSS: u-blox SAM-M10Q concurrent multi-constellation receiver (GPS, GLONASS, Galileo, BeiDou) with integrated antenna; UART interface

- Storage: 4-bit SDIO microSD socket (Molex 1040310811, push-push) powered through a gated rail for zero-leakage sleep

- Vehicular power front-end: 20 V TVS, reverse-polarity Schottky, LC pi-filter, Texas Instruments LMR43620-Q1 automotive-grade buck converter

- Battery management: Microchip MCP73871 single-cell Li-ion charger with integrated system-load sharing and power-path MUX

- Post-regulation: Texas Instruments TPS7A4700 ultra-low-noise parallel LDO for the 3.3 V rail

- Peripheral power gating: Diodes Inc. AP22815 low-Rds(on) load switch, enabled from ESP32 IO18

- User I/O: BOOT and RESET tactile buttons, RGB combo LED (XZCMECBDDGK53W), 3-pin GPIO breakout (GND/IO16/IO17)

- Debug: 8 pad-style test points for 5 V, 3.3 V, V_BAT, V_BAT_IC, BOOT, EN, RXD, TXD

- Mechanical: 85 mm × 46 mm four-layer PCB, three 2.5 mm mounting holes

## 2. Block Diagram

The following prose description captures the signal and power topology. For a drawn block diagram, render from the schematic PDF in datasheet-build/schematic.pdf.

### 2.1 Major Blocks

- Power Input Stage — J1 (12 V tractor) feeds D1 PTVS20VZ1USK TVS → D3 Schottky (reverse polarity) → L1 330 nH + C1/C3/C4 pi-filter → U1 LMR43620MSC5-Q1 buck converter (L2 2.2 µH inductor) → +5 V. J2 USB Micro-B provides an alternate +5 V source tied to the same rail.

- Battery Path — U2 MCP73871-2CCI provides single-cell Li-ion charge management and transparent load sharing between +5 V, battery (J3 → /V_BAT), and system output (/5V_3.7V). TH1 10 kΩ NTC thermistor reports pack temperature. R2 1 kΩ sets a 1 A fast-charge current (per datasheet Rprog).

- Core Regulation — U3 TPS7A4700RGWR ultra-low-noise adjustable LDO steps /5V_3.7V down to +3.3 V using hard-wired feedback taps (6.4 V + 1.6 V tied, 0.2 V + 0.1 V tied to GND per the TPS7A47 programming table — nominal 3.3 V out).

- Peripheral Gate — U4 AP22815AWT-7 load switch gates /EN_SWITCH/V_OUT (switched 3.3 V). Control: ESP32 IO18 drives the EN pin (high = on). FB1 ferrite bead isolates GPS VCC. Pulled up to /EN via R7-R10 on the output side.

- MCU — U5 ESP32-S3-MINI-1U-N8 is permanently powered from +3.3 V. BOOT1 tactile switch pulls IO0 to GND (strap), RESET1 pulls EN to GND.

- LoRa Subsystem — U7 Wio-SX1262 module on SPI: ESP32 IO10=SCK, IO11=MOSI, IO12=MISO, IO8=NSS, IO9=RST, IO14=BUSY, IO15=DIO1, IO13=RF_SW1. Module carries its own antenna footprint.

- GNSS Subsystem — U6 SAM-M10Q on ESP32 UART2: ESP32 IO41=TXD (to module RXD), IO42=RXD (from module TXD). Powered from switched /EN_SWITCH/V_OUT via ferrite bead FB1. I²C, TIMEPULSE, EXTINT, and SAFEBOOT pins are not routed on this revision.

- Storage — J4 microSD, 4-bit SDIO. ESP32 IO4=CLK, IO5=CMD, IO2=DAT0, IO1=DAT1, IO7=DAT2, IO6=DAT3. Card power fed from /EN_SWITCH/V_OUT. Card-detect switch is not wired to the MCU on this revision (see flag in Section 5).

- USB — J2 USB Micro-B carries the ESP32-S3's native full-speed USB: D+ → IO20, D- → IO19. Used for both firmware upload and runtime USB CDC.

- User Expansion — J5 3-pin Phoenix connector exposes GND, IO16, and IO17 for external sensors or digital I/O. Both ESP32 IOs are ADC-capable.

## 3. Mechanical

Board outline: 85.0 mm × 46.0 mm rectangular, four-layer FR-4 (0.2 mm Edge.Cuts stroke). Overall outline coordinates span (90.38, 52.00) mm to (175.38, 98.00) mm in KiCad board space.

Mounting holes: Three 2.5 mm non-plated holes at board corners: lower-left (93.68, 55.20), upper-left (93.68, 94.70), and upper-right (172.18, 94.70). No lower-right hole is present — [DESIGN NOTE: asymmetric mounting may warrant a fourth hole for production revisions].

Board thickness and stackup: [TBD — extract from KiCad project file / fab notes before release].

Overall height: [TBD — measure with tallest component populated; dominant contributors are the u-blox SAM-M10Q module (~9.7 mm per MPN datasheet) and the Wio-SX1262 LoRa module].

### 3.1 Major Component Positions (KiCad board coordinates, mm)

| Ref | Part | X (mm) | Y (mm) | Rot (deg) |
| --- | --- | --- | --- | --- |
| U5 | ESP32-S3-MINI-1U | 141.63 | 70.85 | 90 |
| U6 | SAM-M10Q GNSS | 113.48 | 74.95 | 0 |
| U7 | Wio-SX1262 LoRa | 155.68 | 85.53 | −90 |
| U1 | LMR43620 buck | 155.20 | 63.95 | 180 |
| U2 | MCP73871 charger | 157.98 | 72.64 | −90 |
| U3 | TPS7A4700 LDO | 157.48 | 81.20 | −90 |
| U4 | AP22815 load switch | 150.08 | 91.80 | 90 |
| J1 | 12 V tractor input | 174.78 | 58.20 | 90 |
| J2 | USB Micro-B | 173.48 | 65.20 | 90 |
| J3 | Battery plug | 174.48 | 78.80 | 90 |
| J4 | microSD | 139.99 | 90.58 | 0 |
| J5 | GPIO breakout | 174.48 | 88.20 | 90 |
| BOOT1 | BOOT button | 133.28 | 57.40 | −90 |
| RESET1 | RESET button | 143.28 | 57.40 | 90 |
| LED1 | RGB indicator | 131.87 | 74.00 | 0 |

## 4. Connector Pinouts

All connectors face the right edge of the board except the microSD (J4, top edge). Coordinates are given in Section 3.1.

### 4.1 J1 — 12 V Tractor Power Input

Phoenix Contact 1725656 (2-pin 3.5 mm pitch pluggable terminal socket).

| Pin | Net Name | Function | MCU GPIO | Voltage Domain | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Chassis / 0 V return | — | GND | Common ground |
| 2 | +12V | Tractor battery input (10–14 V nominal) | — | +12 V (raw) | TVS D1 (PTVS20VZ1USK, 20 V), reverse-polarity D3 Schottky, L1 330 nH + C1/C3/C4 pi-filter, then U1 LMR43620 buck |

### 4.2 J2 — USB Micro-B

Amphenol 10118193-0001LF horizontal USB 2.0 Micro-B receptacle.

| Pin | Net Name | Function | MCU GPIO | Voltage Domain | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | +5V | USB bus power | — | +5 V | Merges with U1 buck output on the +5 V rail |
| 2 | /D- | USB Full-Speed D- | IO19 | USB signaling | Direct to ESP32-S3 native USB phy |
| 3 | /D+ | USB Full-Speed D+ | IO20 | USB signaling | Direct to ESP32-S3 native USB phy |
| 4 | unconnected-(J2-ID-Pad4) | USB ID (not used) | — | — | Intentionally unconnected |
| 5 | GND | USB signal ground | — | GND | — |
| 6 | GND | Cable shield / mechanical | — | GND | — |

### 4.3 J3 — Battery Plug

Phoenix Contact 1725656 (same as J1). Accepts a single Li-ion cell or pack; polarity is enforced by the MCP73871 input but the operator must verify orientation at the connector.

| Pin | Net Name | Function | MCU GPIO | Voltage Domain | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Battery negative | — | GND | — |
| 2 | /V_BAT | Single-cell Li-ion positive (2.9–4.2 V) | — | V_BAT | Ties to U2 MCP73871 VBAT pins 14/15/16 (charge + sense) |

### 4.4 J4 — microSD Socket

Molex 1040310811 push-push socket, 4-bit SDIO. Card VDD is gated — see Section 5.4.

| Pin | Net Name | Function | MCU GPIO | Voltage Domain | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | /DAT2 | SDIO DAT2 | IO7 | Switched 3.3 V | 4-bit SDIO |
| 2 | /DAT3 | SDIO DAT3 / CS | IO6 | Switched 3.3 V | — |
| 3 | /CMD | SDIO CMD | IO5 | Switched 3.3 V | — |
| 4 | /EN_SWITCH/V_OUT | Card VDD | — | Switched 3.3 V | Fed from U4 AP22815 output (IO18 gates) |
| 5 | /CLK | SDIO CLK | IO4 | Switched 3.3 V | — |
| 6 | GND | Card ground | — | GND | — |
| 7 | /DAT0 | SDIO DAT0 | IO2 | Switched 3.3 V | — |
| 8 | /DAT1 | SDIO DAT1 | IO1 | Switched 3.3 V | — |
| 9 | /MICRO_SD_CARD/SW_DETECT | Card-detect mechanical switch | — | — | [FLAG — not routed to MCU on Rev 2; net floats. Consider routing to an IO in Rev 3.] |
| 10 | GND | Card shield / ground | — | GND | — |

### 4.5 J5 — GPIO Breakout

Phoenix Contact 1725669 (3-pin 3.5 mm pitch pluggable terminal socket). Intended for external sensor attachment.

| Pin | Net Name | Function | MCU GPIO | Voltage Domain | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Ground reference | — | GND | — |
| 2 | /ESP32-S3-MINI-1U/IO16 | General-purpose I/O | IO16 | 3.3 V logic | ADC2_CH5 capable |
| 3 | /ESP32-S3-MINI-1U/IO17 | General-purpose I/O | IO17 | 3.3 V logic | ADC2_CH6 capable |

## 5. Power Specifications

### 5.1 Input Ranges

| Input | Connector | Nominal | Range (derived) | Notes |
| --- | --- | --- | --- | --- |
| Vehicular DC | J1 +12V | 12.0 V | 10.0 – 14.0 V (per README) | Absolute max limited by D1 TVS clamp (PTVS20VZ1USK stand-off 20 V). LMR43620-Q1 can tolerate up to 36 V steady-state but transients above the TVS clamp will be shunted. |
| USB bus | J2 +5V | 5.0 V | 4.5 – 5.5 V (USB 2.0 spec) | Direct to +5 V rail; no inline protection diode observed — USB VBUS feeds MCP73871 VPCC and IN pins directly. |
| Battery | J3 /V_BAT | 3.7 V | 2.9 – 4.2 V (single-cell Li-ion) | MCP73871 manages charge and load sharing. |

### 5.2 Rail Summary

| Rail | Nominal | Source | Consumers |
| --- | --- | --- | --- |
| +12V | 12 V | J1 (tractor) | D1 TVS, D3 Schottky anode |
| /12V_5V BUCK/PI_IN | ~12 V | D3 cathode → L1 input | Pi-filter input node |
| /12V_5V BUCK/V_IN_BUCK | ~12 V | L1/C3/C4 output | U1 VIN and EN_UVLO |
| +5V | 5.0 V | U1 LMR43620 SW→L2→+5V (or USB VBUS) | U2 MCP73871 inputs, U1 VOUT_BIAS, TP12, +5V decoupling |
| /V_BAT | 2.9 – 4.2 V | J3 battery | U2 VBAT pins 14/15/16, TP13 |
| /5V_3.7V (V_BAT_IC) | ≈ 3.7 – 5.0 V | U2 OUT (power-mux of +5V and /V_BAT) | U3 TPS7A4700 IN/EN, TP14 |
| +3.3V | 3.3 V | U3 TPS7A4700 LDO output | U5 ESP32-S3 3V3, U7 LoRa RF_SW1, U4 load-switch input, R12/R13 pull-ups, LED1, TP15 |
| /EN_SWITCH/V_OUT | 3.3 V (switched) | U4 AP22815 output (gated by ESP32 IO18 → /EN) | J4 microSD VDD, U6 GPS VCC via FB1, pull-ups R7–R10/R15 |
| Net-(U6-VCC) | 3.3 V | FB1 (ferrite) from /EN_SWITCH/V_OUT | U6 SAM-M10Q VCC and VCC_IO |

### 5.3 Protection

TVS (D1, PTVS20VZ1USK): 20 V bidirectional unidirectional TVS on the +12 V rail, SOD-523 package. Clamps automotive load-dump and inductive transients at the input connector.

Reverse-polarity (D3, SS2PH5HM3-84A per stale BOM — Schottky): Schottky diode in series with the positive input. Drops ~0.4 V at the rated current; improves input current transient absorption together with L1 and the bulk input capacitor.

Fuse: [TBD — no inline fuse is present on the Rev 2 schematic. Application note: the downstream harness is expected to provide vehicle-side fuse protection. A polyfuse is recommended for Rev 3.]

Power-good signalling: The MCP73871 /PG open-drain output is pulled to +3.3 V via R4 and sampled at ESP32 IO34.

### 5.4 Peripheral Power Gating

The ESP32 controls a dedicated 3.3 V rail (/EN_SWITCH/V_OUT) through the AP22815 load switch (U4). Driving IO18 high (/EN) enables the rail; driving low disconnects the GPS module and microSD VDD, allowing deep-sleep current to drop to the ESP32's own sleep budget plus the load switch's off-state leakage. FB1 additionally suppresses conducted noise onto the GPS VCC pin.

### 5.5 Key Behavioral Parameters

| Parameter | Value |
| --- | --- |
| Quiescent current, all rails enabled | [TBD — bench measurement required] |
| Deep-sleep current (rail-gated) | [TBD — bench measurement required; README targets single-digit µA class] |
| Peak current (Wi-Fi TX + GPS fix + SD write) | [TBD — README estimate ≈ 150 mA] |
| LMR43620 switching frequency | [TBD — see component datasheet: LMR43620MSC5-Q1RPE] |
| Operating temperature range | [TBD — see component datasheets] |

## 6. Interfaces

### 6.1 USB 2.0 Full-Speed (Firmware + CDC)

Native ESP32-S3 USB phy. No USB-UART bridge IC. Bootloader entry is driven by the BOOT1 button (IO0 low at reset).

| Signal | ESP32 Pin | GPIO | Routed To | Notes |
| --- | --- | --- | --- | --- |
| USB D+ | 24 | IO20 | J2 pin 3 | No series resistor populated |
| USB D- | 23 | IO19 | J2 pin 2 | No series resistor populated |
| VBUS | — | — | J2 pin 1 (+5V) | Feeds system +5V rail |

### 6.2 LoRa SPI (Wio-SX1262 Module, U7)

Full Semtech SX1262 control interface on ESP32 SPI. [FLAG — U7 library symbol pin-name ordering disagrees with the net-label-derived mapping; the table below uses the authoritative schematic net names, which are what firmware must match.]

| Signal (net) | ESP32 Pin | GPIO | Module Function | Notes |
| --- | --- | --- | --- | --- |
| /SCK | 14 | IO10 | SPI clock | SPI master out |
| /MOSI | 15 | IO11 | SPI MOSI | — |
| /MISO | 16 | IO12 | SPI MISO | — |
| /NSS | 12 | IO8 | SPI chip-select (active-low) | — |
| /RST | 13 | IO9 | Module reset (active-low) | — |
| /BUSY | 18 | IO14 | Module busy indicator | Host must poll before issuing commands |
| /DIO1 | 19 | IO15 | Module IRQ | TX/RX done, timeout, etc. |
| /RF_SW1 | 17 | IO13 | RF switch control (TX/RX select on module) | — |

### 6.3 GNSS UART (SAM-M10Q, U6)

u-blox NMEA/UBX protocol over UART. Default baud rate is 38,400 per the SAM-M10Q default configuration; firmware may reconfigure via UBX messages.

| Signal (net) | ESP32 Pin | GPIO | GPS Pin | Direction | Notes |
| --- | --- | --- | --- | --- | --- |
| /TXD | 37 | IO41 | U6 pin 14 (RXD) | ESP32 → GPS | Configuration / commands |
| /RXD | 38 | IO42 | U6 pin 13 (TXD) | GPS → ESP32 | NMEA / UBX output |
| Power (switched) | — | — | U6 pins 2, 17 | — | From /EN_SWITCH/V_OUT via FB1 |

[FLAG — SAM-M10Q I²C (SDA pin 9, SCL pin 12), TIMEPULSE (pin 7 /1SEC), EXTINT (pin 19 /EXTINT), ~SAFEBOOT (pin 8), and ~RESET (pin 18) are not routed on Rev 2. Time-pulse-based timing applications and MCU-driven safeboot recovery would require routing changes.]

### 6.4 microSD (4-bit SDIO)

The ESP32-S3 SDMMC host drives the card in 4-bit native SDIO mode. Pin assignments match the ESP32-S3 SDMMC slot pinout. See Section 4.4 for the J4 pinout.

[FLAG — microSD card-detect (J4 pin 9) is not routed to any MCU GPIO on Rev 2. Firmware cannot detect card insertion/removal.]

### 6.5 Peripheral Enable & Power-Good

| Signal | ESP32 Pin | GPIO | Direction | Function |
| --- | --- | --- | --- | --- |
| /EN | 22 | IO18 | Output (push-pull) | Drives U4 AP22815 EN input — gates the switched 3.3 V rail to GPS and microSD |
| /PG | 29 | IO34 | Input (pulled to +3.3V via R4) | Power-good from MCP73871 charger; asserted low when VSYS is within regulation |

### 6.6 User Expansion (J5)

J5 exposes ESP32 IO16 and IO17. Both are routed directly to the module without series protection — firmware must configure pull direction and protect against over-voltage at the harness level. Both pins are ADC2-capable; IO16 = ADC2_CH5, IO17 = ADC2_CH6.

### 6.7 Boot Strapping and Reset

BOOT1 (B3F-1000 tactile switch) pulls /IO0 to GND; R14 10 kΩ provides the pull-up to +3.3 V. Holding BOOT1 during a reset enters the ESP32-S3 ROM bootloader (serial download / USB DFU). RESET1 pulls /EN to GND; R13 10 kΩ pulls /EN up to +3.3 V.

## 7. Test Points

Eight 2.0 mm and 2.5 mm solder pads, labelled on the silkscreen.

| Ref | Silkscreen | Net | Exposes | Pad Size |
| --- | --- | --- | --- | --- |
| TP3 | RXD | /RXD | GPS → ESP32 UART receive line (IO42) | ⌀2.5 mm |
| TP6 | TXD | /TXD | ESP32 → GPS UART transmit line (IO41) | ⌀2.5 mm |
| TP12 | 5V | +5V | Post-buck / USB 5 V rail | ⌀2.5 mm |
| TP13 | V_BAT | /V_BAT | Battery voltage direct from J3 | ⌀2.5 mm |
| TP14 | V_BAT_IC | /5V_3.7V LDO/5V/3.7V | MCP73871 system output / LDO input | ⌀2.5 mm |
| TP15 | 3.3V | +3.3V | Post-LDO 3.3 V rail | ⌀2.5 mm |
| TP16 | BOOT | Net-(U5-IO0) | ESP32 IO0 boot strap | ⌀2.0 mm |
| TP17 | EN | Net-(U5-EN) | ESP32 EN / hardware reset | ⌀2.0 mm |

## Appendix A — Bill of Materials

Reference-designator-sorted BOM extracted from the Rev 2 schematic (kicad-cli export). Where the generated BOM lacked MPNs, values were back-filled from the project root bom.csv when reference and value agreed.

Source-of-truth note: the project's root bom.csv predates Rev 2 and does not agree with the schematic on several components (it lists SW1/SW2 tactile switches, D4–D6 discrete LEDs, U4 = AP22653, and omits U7/J5 entirely). The table below uses the live netlist reference designators; the root file should be regenerated before manufacture.

| Ref | Value | MPN | Manufacturer | Footprint | Description / Notes |
| --- | --- | --- | --- | --- | --- |
| BOOT1 | B3F-1000 | Omron B3F-1000 | Omron | footprints:SW_B3F-1000 | SPST tactile button (BOOT) |
| C1 | 10 µF | CGA5L1X7R1H106K160AE | TDK | CAPC3216X190N (1206) | 16 V X7R automotive |
| C2 | 4.7 µF | CGA4J1X7R1H475K125AC | TDK | CAPC2012X150N (0805) | 16 V X7R automotive |
| C3 | 4.7 µF | CGA4J1X7R1H475K125AC | TDK | CAPC2012X150N (0805) | 16 V X7R automotive |
| C4 | 100 nF | 08055C104K4Z2A | AVX | CAPC2012X94N (0805) | X7R |
| C5 | 1 µF | UMK212B7105KGHT | Taiyo Yuden | C_0805_2012Metric | X7R |
| C6 | 100 nF | LMK105B7104KVHF | Taiyo Yuden | CAPC1005X55N (0402) | X7R |
| C7 | 22 µF | LMJ316BB7226KLHT | Taiyo Yuden | CAPC3216X190N (1206) | X7R |
| C8 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | 10 V X7R automotive |
| C9 | 4.7 µF | LMK107BJ475KAHT | Taiyo Yuden | C_0603_1608Metric | X5R |
| C10 | 4.7 µF | LMK107BJ475KAHT | Taiyo Yuden | C_0603_1608Metric | X5R |
| C11 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C12 | 100 nF | C0603C104K8RACAUTO | Kemet | C_0603_1608Metric | X7R automotive |
| C13 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C14 | 1 µF | LMK107BJ105KAHT | Taiyo Yuden | C_0603_1608Metric | X5R |
| C15 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C16 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C17 | 100 nF | C0603C104K8RACAUTO | Kemet | C_0603_1608Metric | X7R |
| C18 | 100 nF | C0603C104K8RACAUTO | Kemet | C_0603_1608Metric | X7R |
| C19 | 100 nF | C0603C104K8RACAUTO | Kemet | C_0603_1608Metric | X7R |
| C20 | 1 µF | LMK107BJ105KAHT | Taiyo Yuden | C_0603_1608Metric | X5R |
| C21 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C22 | 10 µF | GCM21BR71A106KE22K | Murata | C_0805_2012Metric | X7R |
| C23 | 1 µF | LMK107BJ105KAHT | Taiyo Yuden | C_0603_1608Metric | X5R |
| D1 | TVS 20 V | PTVS20VZ1USK | Nexperia | D_SOD-523 | Unidirectional 2000 W TVS |
| D3 | Schottky | SS2PH5HM3-84A | Vishay | CR_SMP_DO-220AA_VIS | Reverse-polarity protection |
| FB1 | Ferrite Bead | MI0603L221R-10 | Laird | R_0603_1608Metric | 220 Ω @ 100 MHz, 1 A |
| J1 | 12 V Tractor | 1725656 | Phoenix Contact | PHOENIX_1725656 | 2-pin 3.5 mm pluggable |
| J2 | USB Micro-B | 10118193-0001LF | Amphenol | USB_Micro-B_Amphenol_10118193-0001LF_Horizontal | Horizontal USB 2.0 |
| J3 | Battery plug | 1725656 | Phoenix Contact | PHOENIX_1725656 | 2-pin 3.5 mm pluggable |
| J4 | microSD | 1040310811 | Molex | 104031-0811_MOL | Push-push microSD socket |
| J5 | GPIO header | 1725669 | Phoenix Contact | PHOENIX_1725669 | 3-pin 3.5 mm pluggable |
| L1 | 330 nH | 744383230033 | Würth | WE-MAPI_2510 | Pi-filter input inductor |
| L2 | 2.2 µH | VLS6045EX-2R2N | TDK | L_TDK_VLS6045EX_VLS6045AF | Buck power inductor |
| LED1 | RGB combo | XZCMECBDDGK53W | Sunled | SON4_XZCME_SLD | Multi-color indicator LED |
| R1 | 10 Ω | ERJ-3EKF10R0V | Panasonic | RES_ERJ3EKF10R0V | 1% thin film |
| R2 | 1 kΩ | RK73H1JTTD1001F | KOA Speer | R_0603_1608Metric | MCP73871 Prog — sets fast-charge current |
| R3 | 10 kΩ | RK73H1JTTD1002F | KOA Speer | R_0603_1608Metric | — |
| R4 | 10 kΩ | CRGCQ0603F470R *see note* | [TBD] | R_0402_1005Metric | PG pull-up — MPN not confirmed in current BOM |
| R7 | 10 kΩ | RC0402JR-0710KL | Yageo | R_0402_1005Metric | Pull-up on switched rail |
| R8 | 10 kΩ | RC0402JR-0710KL | Yageo | R_0402_1005Metric | Pull-up on switched rail |
| R9 | 10 kΩ | RC0402JR-0710KL | Yageo | R_0402_1005Metric | Pull-up on switched rail |
| R10 | 10 kΩ | RC0402JR-0710KL | Yageo | R_0402_1005Metric | Pull-up on switched rail |
| R12 | 10 kΩ | [TBD — footprint missing in schematic] | [TBD] | (unpopulated footprint?) | [FLAG — component has no footprint assigned in netlist] |
| R13 | 10 kΩ | RK73H1JTTD1002F | KOA Speer | R_0603_1608Metric | EN / reset pull-up (pulls ESP32 EN to 3.3V) |
| R14 | 10 kΩ | RK73H1JTTD1002F | KOA Speer | R_0603_1608Metric | IO0 / boot strap pull-up |
| R15 | 10 kΩ | RC0402JR-0710KL | Yageo | R_0402_1005Metric | Pull-up on switched rail |
| RESET1 | B3F-1000 | Omron B3F-1000 | Omron | SW_B3F-1000 | SPST tactile button (RESET) |
| TH1 | 10 kΩ NTC | NCU15XH103F6SRC | Murata | R_0402_1005Metric | Pack thermistor (MCP73871 THERM) |
| TP3 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP6 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP12 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP13 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP14 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP15 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.5 mm pad |
| TP16 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.0 mm pad |
| TP17 | Test pad | rct-0c | Keystone/generic | TP_245X165 | ⌀2.0 mm pad |
| U1 | LMR43620MSC5-Q1RPE | LMR43620MSC5-Q1RPE | Texas Instruments | RPE0009A-MFG (QFN) | 36 V 2 A sync buck, automotive |
| U2 | MCP73871-2CCI/ML | MCP73871-2CCI/ML | Microchip | QFN50P400X400X100-21N | Li-ion charger w/ power-path |
| U3 | TPS7A4700RGWR | TPS7A4700RGWR | Texas Instruments | QFN65P500X500X100-21N | Ultra-low-noise 1 A LDO |
| U4 | AP22815AWT-7 | AP22815AWT-7 | Diodes Inc. | IC5_AP22815AWT-7_DIO (SOT-25) | Load switch. [FLAG — root bom.csv lists AP22653QW6-7; schematic/netlist are authoritative.] |
| U5 | ESP32-S3-MINI-1U-N8 | ESP32-S3-MINI-1U-N8 | Espressif | ESP32-S3-MINI-1U_EXP | Dual-core MCU + Wi-Fi/BLE, 8 MB flash, U.FL antenna variant |
| U6 | SAM-M10Q | SAM-M10Q | u-blox | ublox_SAM-M8Q (mechanically compatible) | Multi-constellation GNSS module with integrated antenna |
| U7 | Wio-SX1262 LoRa module | Wio-SX1262 | Seeed Studio | Wio-SX1262 PCB Package | Semtech SX1262 sub-GHz LoRa module |